| MEETING TITLE: | City of Seguin – Cordova Rd Reconstruction TxDOT 30% Drainage Review Meeting |  | DATE: | 07/05/2023 |  |
| --- | --- | --- | --- | --- | --- |
| PROJECT NUMBER: | 0915-46-052 |  |  |  |  |
| Attendees: | TBD TxDOT- Chip Perry, Eddie Gonzalez, Nellie Shannon PD- Jake Powell, John Tyler, Ethan Floyd Guadalupe County- Clay Forister Seguin- Pablo Martinez |  |  |  |  |
| AGENDA TOPICS: |  |  |  |  |
| Cordova Rd |  | Project Overview: - Overview of project limits - Discuss which design storms are used for which crossings Hydrology Discussion: - Overall method - Ultimate conditions, 25 year crossings with free board - Checking up to 100 yr to verify no impacts - Cordova Study vs Hwy 123 Study Hydraulics Discussion: - SH 46/Cordova - Crossing Z - 10 year storm - Temporary - Need 2’ of cover - How will it be contained? Ditches large enough - need downstream grading - what would 25 yr design be? - Area is developing- need to check ditch capacity - Cordova Road - Crossing A - Depressed upstream side details to be in pse - Label streets, culvert on 46, show 46 - Ditch tables needed - Crossing B - Floodplain to edge of existing street - Houses - Careful with WSE, no raise - Modeled two upstream channels separtately-two reaches - Ran east reach first, used wse to set west reach - No history of flood events - Compared to fema model - Bottom line is to not have impacts upstream or downstream - Crossing C - Very flat upstream, no channelization - Similar to A - Crossing D/E - Rational method - Hwy 123 - Crossing F - Proposing to extend existing culverts - Knowing the SH 123 project will reconstruct intersection - On similar timelines - Planning on Cordova being constructed first - SH 123- Chip to supply progress and clarification on timeline - Let date of 9/2025 - Crossing F-1 - Matching TxDOT plan - Explained existing flow path |  |  |
| ACTION ITEMS & TAKEAWAYS: Description |  |  |  |  |
| - Nellie- Jake handled questions on 7/3- Nellie wants to see culvert layouts |  |  |  |  |