| MEETING TITLE: | City of Seguin – Cordova Rd Reconstruction TxDOT 30% Drainage Review Meeting |  | DATE: | 07/05/2023 |  |
| --- | --- | --- | --- | --- | --- |
| PROJECT NUMBER: | 0915-46-052 |  |  |  |  |
| Attendees: | TBD TxDOT- Chip Perry, Eddie Gonzalez, Nellie Shannon PD- Jake Powell, John Tyler, Ethan Floyd Guadalupe County- Clay Forister Seguin- Pablo Martinez |  |  |  |  |
| AGENDA TOPICS: |  |  |  |  |
| Cordova Rd |  | Project Overview: - Overview of project limits - Discuss which design storms are used for which crossings Hydrology Discussion: - Overall method - Ultimate conditions, 25 year crossings with free board - Checking up to 100 yr to verify no impacts - Cordova Study vs Hwy 123 Study Hydraulics Discussion: - SH 46/Cordova - Crossing Z - Temporary crossing to be replaced by SH 46 project. Designing for 10 year storm - Need 2’ of cover over PP pipe - Likely need downstream grading - Area is developing- need to check ditch capacity - Cordova Road - Crossing A - Depressed upstream details to be in pse - Label streets (all DAM’s), culvert on 46, show 46 - Ditch tables needed in pse |  |  |
|  |  | - Crossing B - Floodplain to edge of existing street - Houses - Careful with WSE, no raise - Modeled two upstream channels separately-two reaches - TxDOT agreed with method - Crossing C - Very flat upstream, no channelization - Similar to A - Crossing D/E - Rational method used, <200 AC. - Hwy 123 - Crossing F - Proposing to extend existing culverts - SH 123 TxDOT project will reconstruct intersection - Planning on Cordova being constructed first - SH 123- TxDOT to supply progress and clarification on timeline - Let date of 9/2025 - Crossing F-1 - Matching TxDOT plan location - Cordova project shows the need for larger structure, further coordination needed |  |  |
| ACTION ITEMS & TAKEAWAYS: Description |  |  |  |  |
| - TxDOT to supply SH 123 PSE design schedule - PD to supply culvert layouts with PSE submittal - PD to download TxDOT comments from BlueBeam Studio Session |  |  |  |  |